How to use the tools to change names and GNIS IDs on NHDFlowlines

Charles Bowker

(Hank Nelson explained it and Charles wrote it down)

1/27/2009

To delete a Name and GNIS ID (Set to <Null>)

Under MODIFY  NHD FLOWLINE ATTRIBUTES hit  “Select NHDFlowlines to Edit”.  Select a bunch of lines that all need to be set to <Null>.  Hit “Edit NHDFlowline Attributes”.  Put blanks into the two fields (GNIS_ID and GNIS_Name) in the common record (The top line in the table).  Then change resolution to local and then back to high (You mess with the resolution column to trick the tools into thinking that you changed the resolution, because it doesn’t consider changing Nulls to blanks to be a real change). Hi light common record so it turns blue.  Hit Apply and make sure that everything changes to <Null>.  Hit OK.

If you are deleting flowline names in a waterbody, watch the status table when you hit Apply NHDFlowline Rules.  If the tools add a bunch of records to the status table (Modfy geometry add feature delete feature ect.) that means that some of the flowlines that are being modified extend beyond the edge of the lake.  The tools are splitting the flowlines at the boundary.  Hit Undo NHDFlowline edits and delete the names manually.  If the APs extend more than a meter over the edge of the lake they will be fixed later and if they extend less than a meter over the boundary they should be ignored.

Save NHDFlowline Edits.

When the data is processes to the QC version the blanks are all converted back to <Null>s.

To change to a real name and GNIS ID.

First use the Identify tool to identify a line that already has the correct name and GNIS ID.

Under MODIFY NHD FLOWLINE ATTRIBUTES hit  “Select NHDFlowlines to Edit”.  Select a bunch of lines that all need to be changed to the same name.  Hit “Edit NHDFlowline Attributes”.  Copy the GNIS ID from the Identify window and paste it into the correct field in the common record (The top line in the table).  Key in the name in the correct field in the common record.  Hi light common record so it turns blue.  Hit apply and make sure that everything changes.  Hit OK

Apply NHDFlowline Rules.

Save NHDFlowline Edits.